## High-Level Description

The Personalized Weather Station is a compact embedded environmental monitoring system designed to collect real-time weather data and send it to a user-facing computer application. The system measures temperature, humidity, and atmospheric pressure using a BME280 sensor connected to an ATmega328P-A microcontroller. The board communicates with a computer over USB serial communication, where a Python program receives the data, graphs the readings, estimates short-term trends, classifies the weather condition, and can send phone notifications through ntfy.

The project also serves as a full PCB design exercise. It includes schematic design, PCB routing, BOM generation, manufacturing file preparation, debugging, and documentation. The design is intended to be manufacturable and expandable for future features such as improved prediction, app integration, wireless communication, cloud logging, and enclosure development.

## Purpose

The purpose of the project is to design a smaller and more personalized alternative to oversized weather monitoring systems while also gaining practical experience with embedded hardware and PCB manufacturing. The system is meant to collect useful environmental data and display it in a way that is easy for the user to understand.

·       Monitor environmental conditions in real time.

·       Measure temperature, humidity, and atmospheric pressure.

·       Use a custom PCB instead of a full development-board-only setup.

·       Practice schematic capture, PCB layout, ERC/DRC checks, BOM creation, and manufacturing preparation.

·       Use USB serial communication so a computer program can graph and process the sensor data.

·       Explore prediction logic and phone notifications as future-facing product features.

## Objectives

·       Design a custom PCB for a personalized weather station.

·       Use an ATmega328P-A microcontroller as the main embedded controller.

·       Use a BME280 sensor for temperature, humidity, and pressure measurement.

·       Add the required support circuitry, including reset pull-up, pull-up resistors, crystal oscillator, decoupling capacitors, voltage regulation, USB-C connection, and ISP programming header.

·       Create a BOM and prepare the board for manufacturing.

·       Validate the concept through testing and demonstration software.

·       Use Python to receive USB serial data, graph actual readings, estimate predicted values, classify weather trends, and send ntfy notifications.

·       Document design choices, troubleshooting steps, and lessons learned.

## System Overview

The system is built around the ATmega328P-A microcontroller. The BME280 sensor collects environmental data and sends it to the microcontroller. The board includes supporting components for stable operation, including an external crystal oscillator, load capacitors, reset circuitry, decoupling capacitors, test points, an ISP programming header, USB-C hardware, a CH340C USB-to-serial interface, and a 3.3 V regulator.

The computer-side Python program connects to the board using USB serial communication. Once connected, it reads temperature, humidity, and pressure values, converts the units, graphs the readings, predicts short-term trends, classifies the weather condition, and sends phone notifications through ntfy during the demo.

## Major Components

| Component | Purpose | Notes |
| --- | --- | --- |
| ATmega328P-A | Main microcontroller | Handles sensor communication, processing, and serial output. |
| BME280 | Environmental sensor | Measures temperature, humidity, and barometric pressure. |
| CH340C | USB-to-serial interface | Allows the board to communicate with the computer through USB serial. |
| USB-C Connector | Power/communication interface | Used for connecting the device to the computer; CC resistors support proper USB-C behavior. |
| LD1117S33TR | 3.3 V voltage regulator | Regulates input voltage down to 3.3 V for the MCU/sensor side of the design. |
| Crystal Oscillator + 22 pF capacitors | MCU clock circuit | Provides a more stable clock than the internal oscillator. |
| R1 10 kOhm | Reset pull-up | Keeps the ATmega328P-A out of reset during normal operation. |
| R2/R3 4.7 kOhm | Communication pull-ups | Support reliable sensor communication lines. |
| R4 330 Ohm + LED | Status indicator | Provides a simple visual indication when the output pin is active. |
| C1/C5/C6/C7 0.1 uF, C2 10 uF | Decoupling/bulk capacitance | Helps stabilize power and reduce noise. |
| J2 ISP Header | Programming connector | Used to flash firmware onto the ATmega328P-A. |
| TP1-TP4 | Test points | Allows probing of signals and ground during testing/debugging. |

# Actual Development Path

The project did not stay exactly the same as the original proposal. The original concept mentioned a Raspberry Pi 4, DHT22, GUI display, and future BME280 pressure integration. What actually happened was that the DHT22 was used only as an early proof of concept, and the project moved to the BME280 because it measures temperature, humidity, and pressure in one sensor. The Raspberry Pi/touchscreen-style product idea was also moved away from because it was too large and expensive for a realistic weather station product.

The final direction became a custom PCB with an ATmega328P-A and BME280, connected to a computer by USB serial. The computer application handles the higher-level user interface, graphing, prediction, and phone notifications. This approach keeps the physical product smaller and closer to a real embedded product while still allowing the demo to show live data clearly.

# Testing and Demonstration

Testing focused on proving that the sensing and data processing concept worked and that the PCB design was manufacturable. The BME280 sensor was used to collect environmental data. The Python program received serial data from the device, displayed a simple Tkinter interface, plotted live data using Matplotlib, predicted short-term trends, classified weather behavior, and sent ntfy notifications. The demonstration showed how the user could plug the device into a computer, run the application, and see weather readings and predictions update automatically.

The Python program used a fixed serial port setting for the demo, read comma-separated temperature, humidity, and pressure data, converted Celsius to Fahrenheit, converted pressure from hPa to inHg, graphed actual and predicted values, and sent phone notifications on a cooldown. This made the software useful for showing the project even before a final polished application was created.

# Troubleshooting and Design Challenges

·       The original proof of concept used a DHT22 because it was available and easy to start with, but it was later dropped once the BME280 worked.

·       Some BME280 modules were damaged during early testing because the pins were incorrectly labeled or misidentified, causing wiring mistakes.

·       The initial PCB layout needed review because missing decoupling/bulk capacitors and other support details had to be added.

·       Test points were added so important signals and ground could be probed during validation.

·       USB-C communication was chosen instead of adding Wi-Fi hardware because the project did not need wireless hardware on the PCB for the current demo.

·       A CH340C USB-to-serial interface was included so the board could communicate with the computer application.

·       A 3.3 V voltage regulator was included because the sensor and MCU side needed a stable lower-voltage supply.

·       The design was explored as a 4-layer board to better understand layer stackups, vias, and inner-layer routing, even though the project started from a simpler PCB approach.

·       The final product direction moved away from Raspberry Pi plus touchscreen because that approach was too bulky and costly for the intended product.

# User Documentation:

## Product Purpose

The Personalized Weather Station is a compact PCB-based environmental monitoring device. It measures temperature, humidity, and barometric pressure using a BME280 sensor. The board sends the data to a computer through USB serial communication, and the computer application displays the readings, graphs the data, predicts short-term trends, and can send phone notifications.

## How to Operate

1. 	Plug the device into a computer using the USB-C connection.

2. 	Open the weather station computer application.

3. 	Start monitoring in the application.

4. 	The device will send sensor readings through USB serial communication.

5. 	The application will graph the readings and display the predicted weather trend.

6. 	Phone notifications may be sent through ntfy during demo operation.

## User Interface Elements

·       USB-C connection: connects the board to the computer for power and serial communication.

·       Computer application: shows monitoring status, plots live readings, and displays the weather prediction.

·       Phone notification: sends weather updates through ntfy when the software triggers a message.

·       Status LED: provides a simple visual indicator when the assigned output pin is active.

·       ISP header: used only for programming the ATmega328P-A, not for normal user operation.

·       Test points: used for engineering validation and troubleshooting, not for normal user interaction.

## External Connections Summary

| Connection | Location | Purpose |
| --- | --- | --- |
| USB-C | J1 / USB-C hardware | Computer connection for power and USB serial communication. |
| ISP Programming | J2 | Firmware flashing for the ATmega328P-A. |
| Test Points | TP1-TP4 | Signal and ground probing during validation. |
| Battery/External Power Option | Battery holder or power input hardware | Optional portable or external power source depending on final build configuration. |